PI3HDX412BD

Demo Board Rev.B User Manual
by Ada Yip

Table of Contents

Introduction	3

1	Overview	3

2	Quick Start	4

3	Circuit Description	5

3.1	Mode Selection	5

3.2	I2C Address and I2C Bus	5

3.3	Equalization Setting	5

3.4	Swing and Pre-emphasis Settings	6

3.5	Source Termination Setting	6

3.6	Splitter Enable	6

3.7	Port Selection	7

3.8	DDC Connection	7

3.9	HDCP Authentication	7

3.10	3.3V Power Supply	8

3.11	5V Power Supply	8

3.12	Power Saving Mode	8

4	References	9

5	Appendix A: Demo Board Schematic	9

# Introduction

PI3HDX412BD HDMI1.4b re-driver and splitter can be employed in source as well as sink applications. This user manual describes the components and the usage of PI3HDX412BD Demo Board Rev.A.

# Overview

Figure 1 is the block diagram of Pericom PI3HDX412BD demo board and figure 2 shows the demo board layout. One HDMI plug connector (J101) on PI3HDX412BD demo board is used for plugging a source device, such as a motherboard. An HDMI cable can be connected between an HDMI receptacle connector (J102/3) on the demo board and a sink device, such as an HDMI monitor. In splitter application, two sink devices can be connected. Switches SW101-2 are used to control the output configurations of PI3HDX412BD.

PI5C3257 is used to select desired SCL, SDA, HPD and +5V signals between two sink devices. If splitter feature of PI3HDX412BD is evaluated, user can choose to connect SCL/SDA to EEPROM at reference U103, instead, after loading an EDID content into the EEPROM.

+5V can be employed to the demo board via a mini USB female connector (J104) and/or the source device provided that header pin JP110 is shorted. This +5V is to power up the sideband switch, PI5C3257, and the on-board EEPROM and pull the pull-up resistors of DDC paths to 5V. +3.3V for the power of PI3HDX412BD is employed via header pin JP101.

PI3HDX412BD

J102

Mini USB Connector

Input Port

SW101

HDMI Input Connector

HDMI Output Connectors

O/P Port 2

U101

+5V_S

J104

+5V_USB

J103

+3V3

+5V

O/P Port 1

SW102

J101

U102

PI5C3257

Switches

TMDS

DDC, HPD

U103

EEPROM

+3V3

+5V

Header Pin

JP101

Figure 1: Block Diagram of PI3HDX412BD Demo Board Rev.A

Figure 2: Layout of PI3HDX412BD Demo Board Rev.A

# Quick Start

To start-up the PI3HDX412BD demo board Rev.A, complete the following steps:

- Short header pins JP110 to allow 5V from the source device employing to the demo board;

- Short header pins JP106-7 to allow reading the EDID content from the selected port;

- Short pins 1 and 16 of switch SW102 to enable pin control mode;

- Keep pins 2 and 15 of switch SW102 open to enable output termination;

- Short pins 4 and 13, and pins 6 and 11 of switch SW102 to set equalization to 5dB; leave pins 3 and 14, and pins 5 and 12 open;

- Short pins 6 and 11, and pins 7 and 10 of SW101 to set swing to 500mV;

- Short pins 5 and 12 of switch SW101 and leave pins 4 and 13 open to set pre-emphasis to 2.5dB;

- Leave pins 3 and 14 of switch SW101 open to enable splitter mode;

- Short pins 2 and 15 of switch SW101 to select output port 1 as the primary port;

- Leave pins 1 and 16 of switch SW101 open to enable PI3HDX412BD;

- Supply 3.3V to header JP101 with JP102 as the ground reference;

- Connect a USB adaptor to J104 to supply 5V to the demo board;

- Connect a source device, e.g. motherboard, to HDMI input connector at J101;

- Connect sink device(s), e.g. HDMI monitor, to HDMI output connector(s) at J102-3.

# Circuit Description

# Mode Selection

Output configurations of PI3HDX412BD can be set via I2C or pin control mode. MS pin is used to select between these two modes.

| MS | Control Mode |
| --- | --- |
| 0 | Pin Control Mode |
| 1 | I2C Control Mode |

Table 1: Mode Selection of PI3HDX412BD Demo Board Rev.A

# I2C Address and I2C Bus

If I2C control mode is selected via MS pin, pins 49-52 of PI3HDX412BD are used as I2C address bits, and pins 19 and 20 are used for transmitting SCL and SDA, respectively. External pull-ups may be required for SCL_CTL and SDA_CTL pins when reading/writing I2C. Pull-ups for SCL_CTL and SDA_CTL can be enabled via switch SW102 by shorting pins 3 and 14, and pins 5 and 12, respectively. Pins 4 and 13, and pins 6 and 11 of switch SW102 should be open in this case.

# Equalization Setting

PI3HDX412BD offers various equalization settings from 2.5dB up to 20dB for serving both source and sink applications. Settings below are for references. The settings can be adjusted using switch SW102 on the demo board in pin control mode or selected via I2C control mode.

| EQ2 | EQ1 | Equalization | Input FR4 Trace | Input HDMI Cable |
| --- | --- | --- | --- | --- |
| 0 | M | 2.5 dB | 2 – 8” | 0 – 7m |
| 0 | 0 | 5 dB |  |  |
| M | 0 | 7.5 dB | 5 – 40” | 5 – 15m |
| 0 | 1 | 10 dB |  |  |
| M | M | 12.5 dB |  |  |
| 1 | 0 | 15 dB | 30” or above | 10m or above |
| 1 | M | 17.5 dB |  |  |
| 1 | 1 | 20 dB |  |  |

Table 2: EQ Setting of PI3HDX412BD Demo Board Rev.A

# Swing and Pre-emphasis Settings

Swing and pre-emphasis settings of PI3HDX412BD are designated for fulfilling source eye diagram requirement specified in HDMI specification as well as for transmitting long output trace in sink application. The settings can be selected via switch SW101 on the demo board for pin control mode or set via I2C control mode.

| SW2 | SW1 | EMP2 | EMP1 | Swing | Pre-emphasis |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 500 mV | 0 dB |
| 0 | 0 | 0 | 1 | 500 mV | 1.5 dB |
| 0 | 0 | 1 | 0 | 500 mV | 2.5 dB |
| 0 | 0 | 1 | 1 | 500 mV | 3.5 dB |
| 0 | 1 | 0 | 0 | 450 mV | 0 dB |
| 0 | 1 | 0 | 1 | 450 mV | 1.5 dB |
| 0 | 1 | 1 | 0 | 450 mV | 2.5 dB |
| 0 | 1 | 1 | 1 | 450 mV | 3.5 dB |
| 1 | 0 | 0 | 0 | 550 mV | 0 dB |
| 1 | 0 | 0 | 1 | 550 mV | 1.5 dB |
| 1 | 0 | 1 | 0 | 550 mV | 2.5 dB |
| 1 | 0 | 1 | 1 | 550 mV | 3.5 dB |
| 1 | 1 | 0 | 0 | 600 mV | 0 dB |
| 1 | 1 | 0 | 1 | 600 mV | 1.5 dB |
| 1 | 1 | 1 | 0 | 600 mV | 2.5 dB |
| 1 | 1 | 1 | 1 | 600 mV | 3.5 dB |

Table 3: Swing and Pre-emphasis Settings of PI3HDX412BD Demo Board Rev.A

# Source Termination Setting

Source termination embedded in PI3HDX412BD is used to minimize the reflection due to any impedance mismatch before PI3HDX412BD. For any transmission higher than 2.2Gbps, source termination is highly recommended.

| ROUT_SEL | Pins 2 and 15 of SW102 | Source Termination |
| --- | --- | --- |
| 0 | Short | Disabled |
| 1 | Open | Enabled |

Table 4: Source Termination Setting of PI3HDX412BD Demo Board Rev.A

# Splitter Enable

PI3HDX412BD has the ability to split the signals from one input port into two output ports. It also offers de-multiplexing feature. To enable splitter mode, DR pin is set.

| DR | Pins 3 and 14 of SW101 | Splitter Mode |
| --- | --- | --- |
| 0 | Short | Disabled |
| 1 | Open | Enabled |

Table 5: Splitter Enable of PI3HDX412BD Demo Board Rev.A

# Port Selection

Although splitter mode is enabled, SEL1 pin of PI3HDX412BD is still active to select a primary output port in pin control mode. S pin of PI5C3257 is in synchronized with the SEL1 pin of PI3HDX412BD. Input source will read the EDID content of the primary sink device and deliver video signals at the sink device’s highest supportive timing to both output ports. Thus, both sink devices should accept the same timings. If the sink devices are with different highest supportive timings, one timing format that both sink devices can support should be stored in the EEPROM at reference U103.

| SEL1 | Selected Port of PI3HDX412BD | Selected Port of PI5C3257 |
| --- | --- | --- |
| 0 | Port 1 | Port 0 |
| 1 | Port 2 | Port 1 |

Table 6: Port Selection of PI3HDX412BD Demo Board Rev.A

# Sideband Signal Connection

As PI3HDX412BD does not carry DDC, HPD and 5V paths, these signals are de-multiplexed through PI5C3257 in the demo board schematic. Port selection of PI5C3257 is aligned with that of PI3HDX412BD. Input SCL and SDA from the source device can be connected to PI5C3257 via header pins JP106-7 if user wishes to read the EDID content of the selected sink device. If a pre-defined EDID content is used in the evaluation, input SCL and SDA can be connected to the on-board EEPROM at reference U103 via header pins JP108-9, instead. The EDID content should be stored in the on-board EEPROM in advance.

Figure 3: Sideband Signal Connection of PI3HDX412BD Demo Board Rev.A

# HDCP Authentication

HDCP engine is not implemented in PI3HDX412BD. Thus, HDCP encryption and decryption must be handled by other devices.

# 3.3V Power Supply

+3V3 is employed to header pin JP101 with ground reference at JP102 to power up PI3HDX412BD on demo board Rev.A.

Figure 4: +3V3 Power of PI3HDX412BD Demo Board Rev.A

# 5V Power Supply

+5V is required by the sideband switch and the on-board EEPROM. It can be employed via two methods:

- +5V_S through an input device connected to the input HDMI connector at reference J101 provided that header pin JP110 is shorted;

- +5V_USB through mini-USB adaptor connected to the mini USB2.0 connector at reference J104.

Short

Figure 5: +5V Power of PI3HDX412BD Demo Board Rev.A

# Power Saving Mode

PI3HDX412BD can enter power saving mode by setting OE pin to a low voltage level.

| OE | Pins 1 and 16 of SW101 | Output TMDS of PI3HDX412BD |
| --- | --- | --- |
| L | Short | Power Saving Mode |
| H | Open | Normal Mode |

Table 7: OE Setting of PI3HDX412BD Demo Board Rev.A

Additionally, PI3HDX412BD offers a squelch feature to detect the presence of input TMDS clock signal. The feature is enabled for saving power at default. It can be disabled via I2C.

# References

- High-Definition Multimedia Interface Specification Version 1.4b, HDMI Licensing, LLC, October 11, 2011

- High-Definition Multimedia Interface Specification Version 2.0, HDMI Licensing, LLC, 2013

- High-Definition Multimedia Interface Compliance Test Specification Version 1.4a, HDMI Licensing, LLC, March 4, 2010

# Appendix A: Demo Board Schematic

For clearer view of schematic diagram, please click the PDF file icon on the right.